Environmental Monitoring Program clam biomass data, 2020-2023

Elizabeth Wells, DWR

May 22, 2025

This data was produced to continue the 1975-2019 dataset created in collaboration by USGS and DWR on USGS and DWR monitoring data for Corbicula fluminea and Potamocorbula amurensis.

For that earlier dataset, clams collected by both agencies were measured by USGS staff, who also performed all calculations. This 2020-2023 dataset follows the same processes and calculations as the earlier dataset, in consultation with Dr. Jan Thompson.  The measuring was done by myself and Sarah Pearson of Logical Zoology, who also worked on the 1975-2019 dataset while at USGS, and I performed the calculations in R.

Clam length data was measured to the nearest millimeter and was used to calculate recruitment (clams <2.5 mm) and average shell length data.  The shell length data was combined with length:ash-free dry biomass regressions created by DWR to calculate dry biomass data. Length-biomass regressions from the same sampling event at a site were used wherever possible.  Where a regression was not available for a given site, the Corbicula regression from station D24-L or the Potamocorbula regression from D7-C were used for the same month and year; in a few cases where D24-L did not have a regression for that month, the subsequent month’s regression data from D24-L was used.

The biomass was further combined with species-specific, temperature-dependent filtration rates to obtain maximum filtration rates, based on EMP water quality monitoring data for water temperature.  Filtration rates were then combined with density, average clam siphon diameter, and recirculation constants to obtain a grazing rate that is somewhat lower than the filtration rate to allow for decreased total filtration due to water recirculation.  The grazing data presented here assumes that the clams are grazing 100% of the time, which may not be realistic; transforming this data to different assumptions should be trivial for the data user.

Each row of data in the 2020-2023 dataset is the average of 4 replicate Ponar, each 0.025 m2.  Earlier data (1975-2019) is also averaged across replicates; there were 1 or 2 replicates in 1975 sampling events, 3 replicates from 1976-1995, and 4 replicates from 1996 onwards.

This data is presented in wide format, with one row per sampling event.  If there is no row for a station at a given month and year, it is because no data is available, usually due to not sampling at that site (due to COVID-19, boat issues, etc.).  A record of missed sampling dates is also provided.

Rows with zero data (samples were taken, but no clams were found) are present in this dataset.

This data is QC’d to the best of my ability.  Measuring data was compared with the taxonomic record and specimens re-examined if the total number of clams was more than 5% different from the taxonomic record.  Small discrepancies with the taxonomic record are due to smaller clams disintegrating from repeated handling, to discovery that a preserved clam was actually dead at the time of preservation, or that the original number of clams was not counted precisely.  The taxonomic data has not been corrected in light of any of these discoveries, as the discrepancies were relatively small.

Data units and notes:

Station: EMP or USGS-assigned name

Year: Numeric year

Month: Calendar month name

Latitude: In decimal degrees

Longitude: In decimal degrees

Density: individuals per m2 of benthic surface

Biomass: grams of dry ash-free biomass per m2 of benthic surface

Filtration rate: m3 water per m2 of benthic surface per day

Filtration rate: m3 water per m2 of benthic surface per day

Recruits: number of individuals with shells<2.5 mm wide per m2 of benthic surface

Length: average shell width in mm, measured to the nearest mm before averaging